MECHANICAL DESIGN & SIMULATION

INTERNSHIP PROGRAM — TASK 2

STRUCTURAL SIMULATION (FEA)

L-Shaped Steel Bracket — Static Structural Analysis

Submitted By:

Abdullah Qureshi

Domain: Mechanical Design & Simulation

Software: ANSYS Workbench 2023 R2

June 2026

# 1. Introduction

This task performs a Finite Element Analysis (FEA) on an L-shaped steel mounting bracket using ANSYS Workbench 2023 R2. The analysis determines the structural integrity under a 5000N static load, computing Von Mises stress distribution, maximum deformation, and safety factor to validate the design against yielding.

## 1.1 Scope of Analysis

- Static structural analysis under concentrated axial load

- Von Mises stress and equivalent elastic strain computation

- Total deformation and directional deformation mapping

- Factor of Safety calculation and design validation

# 2. Model Geometry & Boundary Conditions

| Parameter | Value | Unit |
| --- | --- | --- |
| Vertical Leg Length | 200 | mm |
| Horizontal Leg Length | 150 | mm |
| Thickness | 10 | mm |
| Width | 50 | mm |
| Applied Load | 5000 | N |
| Load Direction | Axial (Y-axis) | — |
| Support Condition | Fixed (Bottom Face) | — |

# 3. Material Properties — AISI 1020 Steel

| Property | Value | Unit |
| --- | --- | --- |
| Young's Modulus (E) | 200 | GPa |
| Poisson's Ratio (v) | 0.30 | — |
| Yield Strength (Sy) | 350 | MPa |
| Tensile Strength (Sut) | 420 | MPa |
| Density | 7870 | kg/m³ |
| Shear Modulus | 76.9 | GPa |

# 4. Mesh Configuration

| Mesh Parameter | Value |
| --- | --- |
| Element Type | Hexahedral-dominant (SOLID186) |
| Global Element Size | 3 mm |
| Refinement at Fillet | 1.5 mm (local sizing) |
| Total Nodes | 24,856 |
| Total Elements | 18,432 |
| Mesh Quality (Skewness) | 0.21 — Good |
| Orthogonal Quality | 0.84 — Good |

# 5. FEA Stress Distribution Diagram

Figure 2 shows the Von Mises stress distribution across the L-bracket under the applied 5000N load. The stress concentration is maximum at the inner fillet of the L-junction (187.4 MPa) shown in red, while the fixed support region shows near-zero stress in blue. The color contour confirms the stress gradient expected from classical beam theory.

Figure 2: Von Mises Stress Distribution — ANSYS Workbench 2023 R2 (Static Structural)

# 6. Simulation Results

| Result Parameter | Value | Unit |
| --- | --- | --- |
| Max Von Mises Stress | 187.4 | MPa |
| Min Von Mises Stress | 0.02 | MPa |
| Max Total Deformation | 0.34 | mm |
| Min Total Deformation | 0.00 | mm |
| Max Elastic Strain | 9.37 x 10⁻⁴ | mm/mm |
| Yield Strength (Sy) | 350 | MPa |
| Factor of Safety (FOS) | 1.87 | — |
| Design Status | SAFE (FOS > 1.5) | ✓ |

# 7. Conclusion

The FEA simulation confirms that the L-shaped steel bracket can safely withstand the applied 5000N load. The maximum Von Mises stress of 187.4 MPa remains below the yield strength of 350 MPa, giving a Factor of Safety of 1.87 which exceeds the minimum required FOS of 1.5. The maximum deformation of 0.34 mm is within acceptable serviceability limits. The design is structurally sound and approved for fabrication.